# Complete Setup Guide: Home Assistant & qups-guard2-ha on Debian Trixie (Raspberry Pi 5)

This document provides a step-by-step procedure to set up a clean Debian Trixie installation on a Raspberry Pi 5. It covers installing Docker, deploying Home Assistant Container, setting up Mosquitto MQTT broker, compiling qups-guard2-ha, and integrating the UPS status monitoring system into Home Assistant.

## 1. System Preparation & Toolchain Installation

Update your Debian Trixie system repository index and install the required compiler tools and libraries (libgpiod, libmosquitto, and libcjson).

Bash

# Update repositories and upgrade base packages

sudo apt update && sudo apt upgrade -y

# Install build dependencies and tools

sudo apt install -y build-essential git libgpiod-dev libmosquitto-dev libcjson-dev

Note on Debian Trixie (Pi 5): Debian Trixie utilizes /dev/gpiochip0 as the default GPIO chip device for header pins on the Pi 5.

## 2. Docker Installation & Home Assistant Deployment

Home Assistant Container runs directly on your host network to enable seamless integration with local services.

### Step 2.1: Install Docker Engine

Bash

# Install Docker via official convenience script

curl -fsSL https://get.docker.com | sh

# Add active user to the docker group

sudo usermod -aG docker <USER>

# Activate group changes without rebooting

newgrp docker

### Step 2.2: Launch Home Assistant Container

Bash

# Create directory for persistent HA config

mkdir -p /home/<USER>/homeassistant/config

# Run the official Home Assistant Container

docker run -d \

--name homeassistant \

--privileged \

--restart=unless-stopped \

-e TZ=<TIMEZONE> \

-v /home/<USER>/homeassistant/config:/config \

--network=host \

ghcr.io/home-assistant/home-assistant:stable

## 3. Mosquitto MQTT Broker Setup

Set up a local Mosquitto MQTT broker to handle messaging between qups-guard2-ha and Home Assistant.

### Step 3.1: Install Mosquitto

Bash

sudo apt install -y mosquitto mosquitto-clients

sudo systemctl enable mosquitto

### Step 3.2: Configure Broker Authentication

Bash

# Set a password for <MQTT_USER> (you will be prompted to enter a password)

sudo mosquitto_passwd -c /etc/mosquitto/passwd <MQTT_USER>

sudo chown mosquitto:mosquitto /etc/mosquitto/passwd

# Create custom listener configuration

sudo bash -c 'cat <<EOF > /etc/mosquitto/conf.d/default.conf

listener 1883

allow_anonymous false

password_file /etc/mosquitto/passwd

EOF'

# Restart Mosquitto to apply changes

sudo systemctl restart mosquitto

## 4. qups-guard2-ha Cloning, Configuration & Installation

Clone the source code into /home/<USER>/qups-guard, build the binary, and create the configuration file.

### Step 4.1: Clone and Compile

Bash

# Create target directory

mkdir -p /home/<USER>/qups-guard

cd /home/<USER>/qups-guard

# Clone your repository (or place your qups-guard2-ha.c file here)

# git clone <REPO_URL> .

# Compile the source file

gcc -O2 qups-guard2-ha.c -lgpiod -lmosquitto -lcjson -lpthread -o qups-guard2-ha

# Move compiled binary to system path

sudo mv qups-guard2-ha /usr/local/bin/

sudo chmod +x /usr/local/bin/qups-guard2-ha

### Step 4.2: Create Configuration File

Create /home/<USER>/qups-guard/qups-guard2-ha.json:

Bash

nano /home/<USER>/qups-guard/qups-guard2-ha.json

Paste the following JSON structure (replace <MQTT_PASSWORD> with the password created in Step 3.2):

JSON

{

"gpio": {

"dip": "01",

"chip_path": "/dev/gpiochip0"

},

"ups": {

"shutdown_delay": 10

},

"mqtt": {

"enabled": true,

"broker": "127.0.0.1",

"port": 1883,

"username": "<MQTT_USER>",

"password": "<MQTT_PASSWORD>",

"node_id": "qups_guard",

"state_topic": "qups/state",

"discovery_prefix": "homeassistant"

}

}

Hardware DIP Switch Mapping: Ensure the "dip" string matches your hardware jumper/switch setup:

- "10" >>> PFO: GPIO 17, LIM: GPIO 27, SHD: GPIO 22

- "01" >>> PFO: GPIO 23, LIM: GPIO 24, SHD: GPIO 25 (DEFAULT)

- "11" >>> PFO: GPIO 5, LIM: GPIO 6, SHD: GPIO 26

## 5. Background Service Configuration (systemd)

Run qups-guard2-ha as a system service so that it starts automatically at boot and runs with root permissions for shutdown routines.

- Create the service unit file:

- Bash

- sudo nano /etc/systemd/system/qups-guard2-ha.service

- Add the following contents:

- Ini, TOML

- [Unit]

- Description=qUPS Guard HA Daemon

- After=network.target mosquitto.service

- [Service]

- Type=simple

- ExecStart=/usr/local/bin/qups-guard2-ha --config /home/<USER>/qups-guard/qups-guard2-ha.json

- Restart=always

- RestartSec=5

- User=root

- [Install]

- WantedBy=multi-user.target

- Reload systemd, enable, and start the daemon:

- Bash

- sudo systemctl daemon-reload

- sudo systemctl enable qups-guard2-ha

- sudo systemctl start qups-guard2-ha

- Verify service status and logs:

- Bash

- sudo journalctl -u qups-guard2-ha -f

## 6. Home Assistant Setup & MQTT Integration

- Open your web browser and navigate to http://<SERVER_IP>:8123.

- Follow the onboarding steps to create your primary administrative account.

- In Home Assistant, go to Settings >>> Devices & Services.

- Click Add Integration (bottom right) and select MQTT.

- Configure the MQTT connection settings:

  - Broker: 127.0.0.1 (or <SERVER_IP>)

  - Port: 1883

  - Username: <MQTT_USER>

  - Password: (the password created in Step 3.2)

- Click Submit.

## 7. Verification & Testing

### Step 7.1: Verify Device Auto-Discovery

- Go to Settings >>> Devices & Services >>> MQTT.

- Click on Devices.

- Confirm that AQEX qUPS Guard is listed with the following entities:

  - binary_sensor.qups_mains_power (plug class): ON (Mains power OK) / OFF (Outage).

  - binary_sensor.qups_energy_low (problem class): OFF (Normal) / ON (Battery critical).

  - sensor.qups_status: Text sensor showing real-time UPS status.

### Step 7.2: Test Mains Power Event

- Disconnect the main supply voltage from the qUPS unit.

- Check the live terminal logs:

- Bash

- sudo journalctl -u qups-guard2-ha -f

  - Expected log output: UPS line power NOK!

  - Home Assistant state: binary_sensor.qups_mains_power transitions to OFF.

- Reconnect the power supply.

  - Expected log output: UPS line power OK.

  - Home Assistant state: binary_sensor.qups_mains_power returns to ON.